RIVERS STATE UNIVERSITY NKPOLU-OROWORUKWO

AN ASSIGNMENT  SUBMITTED IN PARTIAL FULFILLMENT OF THE

COURSE:
DIGITAL FILTER THEORY(PHY 822)

BY

NDUBUISI MARVELLOUS CHIOMA

REG NUMBER: MSC.2024/FT/PHY/0003/2024/202211771836

LECTURER

Dr. P.A. NGERI

- OCTOBER 2025

Write short on the following:

NMO Correction

Velocity Analysis

Static  Correction

Continuous and Discrete Signals and sampling interval

- NORMAL MOVEOUT (NMO) CORRECTION

Normal Moveout(NMO) Correction is a fundamental, time-variant correction applied in reflection seismic data processing to prepare seismic traces for the final stacking process.

Purpose

The primary goal of NMO correction is to align reflection events across a common Midpoint (CMP) gather to simulate a zero-offset (ZO) recording.

- Compensation for Offset: in seismic data acquisition, the reflection arrival time (t) increases as the distance between the source and the receiver(the offset, x) increases. This extra travel time, known as Normal Moveout (∆TNMO), is given by the hyperbolic approximation for a flat layer:

- X2

- t2(x) ≈ t20 + –––––––

- V2NMO

- Where t0 is the zero-offset two-way travel time, and VNMO is the Normal Moveout velocity (or stacking velocity)

- Flattening Reflection: the NMO correction is applied by subtracting ∆TNMO from the measured travel time t(x) for each sample:

- Tcorrected = t(x) - ∆TNMO ≈  t0

- This process flattens the hyperbolic reflection curves on the CMP gather, making the reflection event align horizontally at  t0.

- Enhancing Stack Quality: Once the events are aligned, the traces in the CMP gather can be stacked(summed). Stacking constructively adds the aligned reflection signals, which significantly improves the signal-to-noise ratio by attenuating random noise and coherent multiple.

- Velocity analysis

- Accurate NMO correction is entirely dependent on having the correct NMO velocity (VNMO) for each time and depth.

- Process: Velocity Analysis is performed by iteratively testing different velocities. The velocity that maximizes the coherence (or flatness) of the reflection event, typically measured using a function like Semblance, is chosen as the optimal VNMO.

- Significance: the derived VNMO is critical not only for stacking but also serves as the basis for calculating interval velocities (using the Dix equation) used in depth-migration and geological interpretation.

- Limitation: NMO Stretch

- The main limitation of conventional NMO correction is the NMO stretch artifact.

- Effect: this stretching causes the frequency content of the reflection wavelet to be compressed towards lower frequency (a loss of high-frequency resolution), particularly severe at shallow times and large offsets

- Causes: Since the correction involves a time-variant shift that effectively “stretches” the time axis of the trace, it introduces a distortion in the seismic wavelet.

- Mitigation: To minimize this damage, traces that exhibit excessive stretch are typically removed (or zeroed out) using an operation called a Stretch Mute before the final stacking.

- VELOCITY ANALYSIS

- Velocity analysis is a crucial process in seismic data processing used to accurately estimate the seismic wave speed (velocity) within the Earth. The estimated velocity field is vital for performing Normal Moveout(NMO) correction, creating the final stacked seismic section, and eventually converting seismic travel times to geological depths.

- Core Concept: Stacking velocity

- The primary output of velocity analysis is the stacking velocity ( VNMO or Vstack)

- Definition: stacking velocity is the velocity function that, when used in the NMO equation, best aligns the reflection events across a common Midpoint(CMP) gather.

- Purpose: the optimal stacking velocity ensures that when the traces are summed (stacked), the signal energy is coherently added, maximizing the amplitude of the reflected event and suppressing noise.

- Relationship to True Velocity: for a horizontal, flat reflector, the stacking velocity is approximately equal to the Root Mean Square(RMS) velocity of the layers above the reflector. This RMS velocity  can then be used with the Dix equation to calculate the interval velocity (the true velocity of each layer).

- How it is Performed

- Velocity analysis is typically an iterative, interactive process performed on sorted CMP gathers:

- Generate a Velocity Spectrum: for a selected CMP gather and a specific two-way zero-offset time (Vtrial) is tested. For each Vtrial, the seismic energy is summed along the corresponding hyperbolic travel time curve.

- Coherence measurement(Semblance):A coherence function, most commonly Semblance, is used to measure how well the traces align. Semblance is the ratio of the stacked energy to the energy of the input traces, ranging from 0 (no alignment) t0 1 (perfect alignment).

- Picking: the results are displayed on a velocity spectrum (a color map of coherence as a function of t0 and Vtrial). the interpreter manually or automatically picks the peak of coherence on the spectrum, which defines the optimal Vstack at that particular time.

- Application: the selected Vstack is then used to apply the NMO correction to flatten the event before stacking the traces.

- Importance

- Accurate velocity analysis is indispensable because velocity is the only link between the recorded seismic travel time and the desired spatial dimensions of the subsurface structure. Errors in Vstack directly lead to:

- Poor quality: low-resolution stacked images.

- Incorrectly defined geological structures (especially when using migration techniques).

- Inaccurate depth estimates.

- STATIC CORRECTION(STATICS)

- Static correction (or statics) is an essential initial step in seismic data processing that applies a bulk, time-invariant shift to entire seismic traces. The purpose is to remove time delays caused by near-surface irregularities, ensuring that all reflection events are time-referenced to a single, flat reference datum.

- Sources of error

- Statics compensate for two main sources of unwanted travel time variations:

- Elevation(Topography) Variations: Difference in the elevation of seismic sources (shots) and receivers (geophones) along the survey line can cause corresponding variations in wave travel time.

- Weathered layer (Low-velocity layer - LVL) Variations: the shallowest layer of the Earth’s surface is often unconsolidated, highly weathered material with a much lower seismic velocity(Vw) than the underlying consolidated rock. Variations in the thickness or velocity of this layer cause significant, short-wavelength time shifts.

- Correction procedure

- The correction aims to calculate the time difference (∆tstatic) between the casual recording surface and the theoretical flat reference datum and then shift the entire trace by that amount.

- The overall static correction for a trace is the sum of the source static and the receiver static:

- ∆tstatic =  ∆tstatic     +  ∆treceiver

- Static corrections are typically divided into two categories:

- Field (or Elevation) Statics: these are the initial, large corrections calculated using konwn surface elevetions and the low-velocity layer model(often determined byrefraction statics or uphole surveys).

- Residual statics: these are small, data-driven corrections applied later in the processing sequence. They compensate for minor, time-shift errors that remains after the field statics, often by analyzing the coherence of reflection events in common midpoint (CMP) gathers.

- Importance:

- Applying statics is crucial because it ensures reflections from the same subsurface point align correctly. Without proper static correction , reflections would appear jagged, hindering velocity analysis, preventing effective NMO correction, and severely degrading the quality of the final stacked seismic image.

- CONTINUOUS AND DISCRETE SIGNALS AND SAMPLING INTERVAL

Signals are fundamentally classified based on the nature of their independent variable (ussually time). the two primary classification are continuous-Time and Discrete-Time signals.

Continous-Time signals

A continuous-time signal, denoted as x(t), is a signal whose independent variable, t, is a continuum (defined for every real value over an interval).

Definition: the signal has a value at every single instant of time.

Domain: Continous(e.g., t

Graphical Representation: A smooth, unbroken curve or waveform

Examples(Analog signals):

- Voltage from battery.

- Speech or music captured by an analog microphone.

- The temperature reading from a mercury thermometer over a day.

Discrete-Time signal

A Discrete-time Signal, denoted as x[n], is a signal whose independent variable, n, is an integer(defined only at specific distinct points in time).

Definition: the signal is a sequence of values; it is not defined between the sampled points.

Domian: Discrete(e.g., n ∈Ζ).

Graphical Representation: A sequence of vertical lines or dots (stem plot) at each discrete time index.

Examples:

- Stock amrket closing prices recoreded daily

- Digital audio created by sampling an analog signal.

- The average monthly rainfall recorded over a year

Key  Relationship: Sampling

Discrete-time signal are often created from continuous-time signals through a process called SAMPLING.

An Analog-to-Digital converter (ADC) measures the amplitude of the continuous signal at uniform intervals, t(the sampling period), generating a sequence of values:

X[n] = x(nT)

This process is the foundation of Digital processing(DSP).

Sampling Interval (Sampling Period)

The sampling interval(denoted as T, ∆t, or Ts) is the fixed time period or distance between successive measurements(samples) taken from a continuous physical signal or a statistical population

In Signal Processing

In digital signal processing(DSP), an analog (continuous-time) signal is converted into a discrete-time signal by taking measurements at regular time intervals

- Definition:

The sampling interval(T) is the exact time duration between two consecutive samples. For example, if a signal is sampled every 0.001 seconds, the sampling interval is T = 1 ms.

- Relationship to Frequency:

The sampling intervals is the reciprocal of the sampling frequency or sampling rate(Fs):

- Importance (Nyquist-shannon Theorem):

The selection of the sampling interval is critical to avoid aliasing where high-frequency components of the signal appear as lower-frequency components in the sampled data. According to the Nyquist theorem, the sampling frequency(Fs) must be at least twice the maximum frequency component (Fmax) of the signal being sampled (Fs = 2Fmax).

This corresponds to a maximum permissible sampling interval of:

1

T ≤  ––––

2Fmax

In research and statistics

In statistics and survey methodology, the sampling interval is used in systematic sampling.

- Definition:

The sampling interval is the fixed ratio used to select elements from a population list. It is calculated by dividing the total population size (N) by the desired sample size (n):

N

Sampling interval = ––––

n

- Application:

If a population has 10,000 people and a researcher wants a sample of 1000, the sampling interval is:

10,000/1,000 = 10

The researcher would then select every 10th person from a randomized list after a randomly selected starting point. This technique ensures a representative sample is drawn from the entire population.

In the context of Analog-to-Digital conversion

The sampling interval and sampling rate/frequency are closely related and play a vital role in determining how accurately an analog signal is represented in digital form.